# Name: Habiba Mohamed ElShahat

# H-Bridge DC Motor Driver Using Bipolar Junction Transistors (BJTs)

## 1. Objective

The purpose of this circuit is to control the direction of a DC motor by reversing the polarity of the voltage applied to it. This is achieved using an H-bridge configuration with discrete BJTs and flyback diodes for motor protection.

## 2. Circuit Description

The circuit consists of four transistors (Q1–Q4) arranged in an H-pattern around the motor. The switching of these transistors controls the current path through the motor, enabling forward or reverse rotation.

Main sections:

1. Power Supply (BAT1): 12 V DC source powers the motor and transistor network. Suitable for most small-to-medium DC motors.

2. Switch Control (SW1): SPDT switch selects which diagonal pair of transistors is activated.

3. Transistors (Q1 & Q3 PNP, Q2 & Q4 NPN): High-side and low-side switching for motor polarity control.

4. Base Resistors (R1–R4, 1 kΩ): Limit base current and ensure transistor saturation.

5. Flyback Diodes (D1–D4): Protect against voltage spikes from the motor's inductance.

6. Motor: Connected between the midpoints of the transistor pairs.

## 3. Working Principle

The SPDT switch selects which diagonal pair of transistors turns ON:
- Forward: Q1 + Q4 ON, Q2 + Q3 OFF.
- Reverse: Q2 + Q3 ON, Q1 + Q4 OFF.
Only one diagonal is ON at any time to avoid shoot-through.

## 4. Component Selection Justification

| Component | Value / Type | Reason for Selection |
| --- | --- | --- |
| Q1, Q3 | 2N2905 (PNP BJT) | High-side switching, easy base drive, moderate current handling. |
| Q2, Q4 | 2N2222 (NPN BJT) | Low-side switching, reliable, low cost. |
| D1–D4 | General-purpose diode | Handles reverse voltage spikes from motor inductance. |
| R1–R4 | 1 kΩ | Limits base current while ensuring saturation. |
| SW1 | SPDT switch | Manual control of motor direction. |
| BAT1 | 12 V DC | Matches common DC motor ratings. |

## 5. Advantages

- Simple and easy to understand for learning.
- Uses widely available, low-cost components.
- Provides reliable direction control with over-voltage protection.

## 6. Limitations

- Manual switch control, no speed control.

- Speed control can be integrated using a microcontroller and a PWM pin

- BJT design has higher voltage drop compared to MOSFETs.
- Not suitable for high-current motors without higher-rated components.