Name: Habiba Mohamed ElShahat

NPN & PNP overprotection circuits

1. Overview

- Left Circuit: NPN-based low-side control using two NPN BJTs (Q1 and Q2 — 2N2222).

- Right Circuit: PNP-based high-side control using a PNP transistor (Q3) to drive a PNP pass transistor (Q4 — 2N2905) with Zener voltage sensing.

Both aim to protect the load from excessive supply voltage by cutting off current flow when the supply exceeds a certain threshold.

2. Circuit Operation

2.1 NPN Low-Side Overvoltage Protection (Left Circuit)

Main Components:

- Q1: NPN transistor (voltage sensing stage)

- Q2: NPN transistor (switching stage to control the load)

- R1, R2, R4: Voltage divider and biasing network

- Load: 1 kΩ resistor

- Supply: 12 V

Working:

- The voltage divider R1–R4 sets a bias voltage at the base of Q1.

- When the supply voltage exceeds the design threshold, Q1 turns ON.

- Q1’s conduction pulls Q2’s base low, turning OFF Q2 and disconnecting the load from ground (low-side switch).

- Below the threshold, Q1 is OFF, allowing Q2 to remain ON, powering the load.

2.2 PNP High-Side Overvoltage Protection (Right Circuit)

Main Components:

- D1: Zener diode (1N4728A — 3.3 V breakdown voltage)

- Q3: PNP transistor (sensing stage)

- Q4: PNP transistor (pass element controlling the load)

- R5, R3, R6: Bias resistors

- R7: Load resistor (100 Ω)

- Supply: 15 V

Working:

- Zener D1 is reverse-biased; it stays OFF until the supply voltage exceeds the Zener breakdown voltage + base-emitter drop of Q3.

- Above threshold, the Zener conducts, applying a voltage to Q3’s base.

- Q3 turns OFF, which in turn removes base drive from Q4, cutting power to the load.

- Below threshold, Q3 is ON, supplying base current to Q4, and the load is powered.

3. Comparison

| Feature | NPN Low-Side Circuit | PNP High-Side Circuit |
| --- | --- | --- |
| Switching Position | Low side (between load and ground) | High side (between supply and load) |
| Voltage Sensing | Resistive divider to NPN base | Zener diode to PNP base |
| Load Reference | Disconnected from ground when tripped | Disconnected from Vcc when tripped |
| Complexity | Slightly more components (2 BJTs, divider) | Fewer transistors (2 PNP + Zener) |
| Current Capability | Limited by Q2’s rating | Limited by Q4’s rating |
| Simulation Behavior | Floating load → unrealistic 88.8 V read | Floating load → unrealistic 88.8 V read |
| Real-World Use | Easier for ground-referenced switching | Preferred for keeping load referenced to ground |

4. Key Observations from Simulation

- Trip Point Setting — In the NPN version, it’s determined by the R1–R4 ratio and Q1’s VBE. In the PNP version, it’s determined by Zener voltage + VBE.

- Load Safety — Both designs successfully remove supply from the load above the set voltage.

- Design Flexibility — The PNP + Zener method is easier to set precisely by just changing the Zener value, while the NPN divider method requires recalculating resistor ratios.

5. Conclusion

- Functionality: Both circuits achieve overvoltage protection by cutting off the load when voltage exceeds a threshold.

- Implementation:

  - NPN Low-Side is good for simple circuits where disconnecting the ground is acceptable.

  - PNP High-Side is better for maintaining a grounded load, especially in automotive or sensitive electronics.

- Recommendation: For practical designs, the PNP High-Side with Zener is more robust and predictable because the trip voltage is set mainly by the Zener diode, making tuning easier.